|  |  | DC RESISTIVE LOAD BANK 440V/20A |  |
| --- | --- | --- | --- |
| We have attained a matchless position in the market as a Leading Manufacturer and Supplier of Load Banks. These are highly appreciated by our clients for their use in Power Supply Equipment, Standard Generator Testing, Battery Discharge, Elevator, Frequency Converters, UPS power and more. The offered banks are used for single or three phase testing of DC or AC supplies with complete accuracy. Their electricity hazard protected mechanism is completely safe to operate. Moreover, our Load Bank has a weather resistant body and has extended operational life. |  |  |  |  |  |  |
| Note: *Above shown image is for representation purpose only. RETRACK SYSTEMS #1038,’Hombelaku’, 11th main, Hampinagar, Vijayanagar 2nd Stage, Bangalore – 560 040. Mob:98447 59090 / 98863 59090 Web-www.retracksystems.com Mail-sales@retracksystems.com/ retracksystems@gmail.com/ retrack_systems@yahoo.com Range of our other products: |  |  |  | TECHNICAL SPECIFICATION. Voltage:; 440V DC. Current:; 20A DC. No of Steps:; 4 Steps (5Ax4). Selection Type:; Suitable DC MCBs. Indications. Separate indicators for Voltmeter and Current Meters. FEATURES. Type of cooling:; Forced Air, using FAN. Enclosure type:; The Load Bank will be Constructed with rugged M.S Sheets/Mess. Painting:; Powder coating,(Black). Auxiliary Supply:; 230V AC With suitable 3pin socket on door for connecting Aux, AC Supply for cooling fans and meters. Connection Terminals:; Suitable Bus bar. Mobility:; Freely movable through suitable wheels. Approx. Dimension: 450 X 450 X 700 mm (Width x Depth x Height). RUGGED AND AESTHETIC MECHANICAL AND ELECTRICAL DESIGN. SALIENT FEATURES: NON-CORROSIVE PROPERTY STABLE & RELIABLE FUNCTIONING HIGH LOAD STORAGE CAPACITY RELIABLE & SAFE. |
| 500A Regulated Power supply Decade RLC Boxes Auto Transformers R/L/C Load Bank Mil Grade Power Supplies \ CUSTOM MADE LOAD BANKS & POWER SUPPLIES ARE ALSO AVAILABLE |